E2 - Getting started with the Development ecosystem for PSOC™ Edge E84 training manual

About this document

Scope and purpose

This training manual provides a comprehensive guide to getting started with the PSOC™ Edge E84 MCU, including detailed instructions on creating, configuring, building, and running application code examples using ModusToolbox™.

Table of contents

About this document	1

Table of contents	2

1	Introduction	3

2	Pre-requisite	4

3	Required development tools	5

4	Code example: Hello world + Blinky	6

4.1	Objective	6

4.2	Description	6

4.3	Flow chart	6

4.4	Hardware diagram	7

4.5	Project creation	8

4.5.1	Output	9

4.6	Configure/rename a pin using Device Configurator	9

4.7	Conclusion	14

5	Code example: IPC semaphore using PDL	15

5.1	Objective	15

5.2	Description	15

5.3	Flow chart	16

5.4	Hardware diagram	17

5.5	Project creation	17

5.6	Enabling retarget-io on CM55	18

5.6.1	Output	23

5.7	Implement the GPIO button and print messages using both CPUs	23

5.7.1	Output	25

5.8	Use IPC semaphore to synchronise access to UART	25

5.8.1	Output	31

5.9	Conclusion	31

6	Appendix A: Creating a PSOC™ Edge application in ModusToolbox™	32

7	Appendix B: KIT_PSE84_EVAL details	35

Revision history	36

Disclaimer	37

# Introduction

PSOC™ Edge E84 series of Arm® Cortex®-M MCUs feature high-performance, low-power, secured MCUs with advanced machine learning (ML) acceleration for next-generation AI/ML applications.

The PSOC™ Edge E84 MCUs are based on Arm® Cortex®-M55, with Helium DSP support paired with Ethos-U55 NPU as well as a low-power Cortex®-M33 paired with Infineon’s ultra-low power NNLite hardware accelerator and advanced HMI interfaces, including graphics.

This introductory course to the PSOC™ Edge development ecosystem delivers a thorough overview of its software and hardware tools, with detailed, step-by-step guidance to help you quickly build your first application.

# Pre-requisite

Install the software and obtain the hardware listed in the Required development tools section.

This is an introductory training to PSOC™ Edge and ModusToolbox™; however, it is not intended to cover all basic concepts.

For an introduction to PSOC™, including a getting started guide to ModusToolbox™, go to:

https://www.infineon.com/product-information/psocdeveloper

For PSOC™ Edge trainings, from beginner tutorials to advanced trainings, go to:

PSOC™ Edge E84 Training Collection

# Required development tools

ModusToolbox™ software v3.6 or later

Recommended installation via ModusToolbox™ Setup tool

Eclipse IDE for ModusToolbox™ v2025.4.0 or later

Recommended installation via ModusToolbox™ Setup tool

Edge Protect Security Suite v1.6 or later

Installed by ModusToolbox™ Setup tool as a dependency to ModusToolbox™ v3.6

ModusToolbox™ Programming Tools v1.5.0 or later

Installed by ModusToolbox™ Setup tool as a dependency to ModusToolbox™ v3.6

Board support package (BSP):

KIT_PSE84_EVAL_EPC2: v1.0.0 or later (available with ModusToolbox™ v3.6)

Serial terminal emulator

Eclipse IDE for ModusToolbox™ includes a serial terminal. Other options include Tera Term, PuTTY, and so on

PSOC™ Edge E84 Evaluation Kit (KIT_PSE84_EVAL)

Ensure BOOT SW (SW6) should be in 'High'/ON position

For KIT_PSE84_EVAL rev. G, BOOT SW pin is BOOT.1 (P17.6)

Ensure J20 and J21 should be in the Tristate/Not-Connected (NC) position

See Appendix B: KIT_PSE84_EVAL details for more details

# Code example: Hello world + Blinky

## Objective

This code example shows how to implement simple UART communication by printing a Hello world message on a terminal. It also blinks an LED using a timer resource in the PSOC™ Edge E84 MCU. The code example uses the Peripheral driver library (PDL) and Device Configurator included in ModusToolbox™.

## Description

This code example demonstrates the following algorithm flow:

Step 1: Initialise peripherals such as UART and GPIO

Step 2: Print “Hello World” on the serial terminal using UART

Step 3: Blink the LED through the GPIO

## Flow chart

Figure 1 explains the flow of the PSOC™ Edge MCU: Hello world project:

Figure 1	Flow chart of Hello World project

## Hardware diagram

Below is the hardware block diagram for this example:

UART pins:

P6_7 (UART_TX)

P6_5 (UART_RX)

GPIO LED pin:

P16_7 (LED)

This example requires BOOT SW in the ON position.

See Appendix B: KIT_PSE84_EVAL for more details.

Figure 2	Hardware diagram for Hello World example

## Project creation

Follow steps 1-4 in Appendix A: Creating a PSOC™ Edge application in ModusToolbox™ to create a new application

When creating the application, select the PSOC™ Edge Hello World application under the Getting Started section

Figure 3	Selecting application

- Run the application by clicking on the Run option in the Quick Panel. Note that this step will also build the project

Make sure the top level of the project is selected in the project workspace to ensure all three applications are built and programmed. If one of the three applications (cm33_s, cm33_ns, or cm55) is selected, then ModusToolbox™ will build and program the selected application.

Figure 4	Build and program the application

### Output

After the program runs successfully, open the serial terminal and configure the baud rate to 115200-8-N-1. Then press the kit reset button. A message is printed as in Figure 5, and the kit LED blinks at approximately 
1 Hz.

Figure 5	Output of the Hello World program

## Configure/rename a pin using Device Configurator

The Device Configurator provides a graphical view of device peripherals, and it generates macros, data structures, and initialization functions based on your selections. The BSP function cybsp_init() calls the generated functions to set up the clocks, pins, and internal routing. It is typically called from the main() function before using on-chip peripherals such as serial blocks and timer/counters.

When you launch the Device Configurator from Eclipse, you are opening the project’s design.modus file, which is responsible for holding all the BSP configuration information. It contains the following:

Selected device

Resource parameters

Constraints

- Now, let us look at how to configure or rename the pins using the Device Configurator.

- Open the Device Configurator tool that can be found in the Quick Panel area under the BSP Configurators section

Figure 6	Opening Device Configurator

- [INFO] The Device Configurator includes the following tabs:

Peripherals: configures peripherals, including communication, digital and system

Analog: configures analog modules, including the autonomous analog, which includes ADC, PTComps, autonomous controller, CTBs, and PRB; and the low-power comparators

Pins: configures the GPIOs, including Smart IO functionality

Clocks: configures the device clock tree, including all clock inputs and outputs

System: configures security, protection, and system settings such as debug and power modes

Memory: configures the memory allocation and provides a graphical view of the memory map

DMA: configures the device’s DMAs

Feel free to navigate through the different tabs. The Device Configurator will only generate code when the design.modus file is saved.

- In the Device Configurator, go to the Pins tab

Figure 7	Pins tab in Device Configurator

- [INFO] In this particular development kit, the three user LEDs are connected to Port 16, pins 5, 6 and 7, as per the kit schematic.

Figure 8	Connection of LEDs in EVK schematics

Each pin can be uniquely identified by giving it a name of your choice. These names are known as aliases. In the code, aliases can be used to refer to pins instead of using explicit PINs.

Figure 9	Pin alias in Device Configurator

- Rename Port 16[7] as TRAINING_LED. We will be using this LED in our application

- Select File→Save or press Ctrl+S to save the change and regenerate source files.

Figure 10	Renaming P16[7] as TRAINING_LED

- [INFO] Changes made in the Device Configurator are enabled once you save the file. 
Try opening the cycfg_pins.h file (located at <application-directory>\bsps\TARGET_APP_KIT_PSE84_EVAL_EPC2\config\GeneratedSource\cycfg_pins.h), and you should be able to observe the pin definitions and aliases. 
The changes we have made are reflected in the file as in Figure 11:

Figure 11	Code generation from Device Configurator

- [INFO] The Device Configurator includes links to documentation. Click the link in the Parameters window to open and navigate through the GPIO documentation. Alternatively, the documentation is also available in PSOC™ E8XXGP Device Support Library

Figure 12	Documentation for GPIO PDL

[INFO] In the Project Explorer, you should be able to observe three folders named PSOC_Edge_Hello_World.proj_cm33_ns, PSOC_Edge_Hello_World.proj_cm33_s, and PSOC_Edge_Hello_World.proj_cm55
Each folder contains a main.c file which contains application code for each corresponding project.

Open the main.c file under PSOC_Edge_Hello_World.proj_cm33_ns
We will modify the code to align with the new LED alias

- Inside the main.c file, under the infinite for loop, change the CYBSP_USER_LED_PORT to TRAINING_LED_PORT and CYBSP_USER_LED_PIN to TRAINING_LED_PIN as shown in code lisitng 1.

Code listing 1	Modification for Hello World application

for(;;) { Cy_GPIO_Inv(TRAINING_LED_PORT, TRAINING_LED_PIN); Cy_SysLib_Delay(BLINKY_LED_DELAY_MSEC); }

- Save the file. We are done with adding the application code for proj_cm33_ns

- Rebuild by clicking on the Build Application option in the Quick Panel

- After the build is successfully completed, program the Evaluation kit by clicking the Launch button in the Quick Panel

- After the program runs successfully, open the serial terminal and configure the baud rate to 115200-8-N-1. Then reset the kit. The same application should be executed, and the kit LED should continue blinking at approximately 1 Hz

## Conclusion

We successfully created our first PSOC™ Edge E84 MCU application using ModusToolbox™, modified code using PDL libraries, printed a message in a terminal, and blinked an LED.

This confirms that the device, EVK, and tools are functional.

# Code example: IPC semaphore using PDL

## Objective

This lab exercise demonstrates how to use the inter-processor communication (IPC) PDL driver to implement a simple semaphore scheme with PSOC™ Edge. A semaphore is used to manage access to a common resource, such as a UART, between the CPUs.

This guide utilizes the Hello World example to show the step-by-step process to add IPC semaphores and shows features like the Library Manager and software documentation; however, ModusToolbox™ includes the mtb-example-psoc-edge-ipc-sema code example, which showcases this functionality in a ready-to-use project.

## Description

A shared IPC semaphore is used to coordinate access to a common resource between the two CPUs. In this example, the resource is the UART: each CPU prints a message to the computer terminal over the UART whenever the user button is pressed. Before printing, both CPUs try to acquire the semaphore. If successful, the CPU prints the message and then releases the semaphore. If the semaphore is already held, it keeps trying until it can acquire it.

Without the semaphore to synchronize access to the UART, messages from both CPUs will overlap. See the flowchart to see visually how the code will operate.

## Flow chart

Figure 13	IPC semaphore code example flow

## Hardware diagram

Below is the hardware block diagram for this example:

UART pins:

P6_7 (UART_TX)

P6_5 (UART_RX)

GPIO LED pin:

P16_7 (LED)

Buton pin:

P8_3 (USER_BTN1)

This example requires BOOT SW in the ON position.

See Appendix B: KIT_PSE84_EVAL details for more details.

Figure 14	Hardware diagram for Hello World example

## Project creation

This lab utilizes the same hello world example from Code example: Hello world + Blinky. Alternatively, create a new Hello World example and name it differently.

## Enabling retarget-io on CM55

The Hello World example utilizes the retarget-io library to print debug information from the CM33 project using UART. In this section, we will show how to use the Library Manager to do the same using the CM55 core.

- Open the Library Manager from the Quick Panel

The Library Manager provides a GUI to select which Board Support Package (BSP) should be used when building a ModusToolbox™ application. The tool collects a list of available and currently selected BSPs and libraries, as well as all the necessary metadata from a webservice. The tool allows you to add and remove BSPs and libraries, as well as change their versions.

Figure 15	Opening Library Manager

- [INFO] Observe that the project already includes multiple libraries for the CM33 secure, CM33 non-secure, and CM55 projects. Libraries worth highlighting for this example are:

mtb-dsl-pse8xxgp: provides all necessary device-specific components to be used during software development for the PSE8xxGP family of devices, including HAL and PDL drivers

mtb-ipc: allows communication between multiple CPUs or between multiple tasks operating in different domains within a single CPU

retarget-io: retargets the standard input/output (STDIO) messages to a UART port

mtb-dsl-pse8xxgp is included for all PSOC™ Edge examples in ModusToolbox™.

This lab requires mtb-ipc on proj_cm33_ns and proj_cm55 since we will communicate between both cores. Observe that the library is enabled for both projects.

On the other hand, retarget-io is enabled for proj_cm33_ns and not for proj_cm55. This library is not required for IPC semaphores; however, we will show how to move this library to utilize printf in proj_cm55.

- In the proj_cm33_ns, click the cross icon to remove the retarget-io, since we will not be using this library on the CM33. Click Add Library

Figure 16	Remove retarget-io from proj_cm33_ns

- Select proj_cm55 as the Target Project, then enable the retarget-io library by clicking on the checkbox, and click OK

Figure 17	Add the retarget-io library to proj_cm55

- In the main Library Manager window, observe that the retarget-io library was removed from proj_cm33_ns and added to proj_cm55, then click Update. This will effectively add the library to the project

Figure 18	Update libraries in project

- Close the Library Manager after completing the update process

- The cm33_ns project includes retarget_io_init.c/h files to initialize the retarget-io library. These files can be reused for the cm55 project by either cutting-and-pasting them, or drag-and-dropping them in the project explorer

Figure 19	Move retarget_io_init files from proj_cm33_ns to proj_cm55

- Open proj_cm33_ns/main.c and modify it to avoid using retarget-io and printf

Code listing 2	Remove retarget_io_init.h from cm33_ns

/******************************************************************************* * Header Files *******************************************************************************/ #include "cybsp.h" //#include "retarget_io_init.h"

Code listing 3	Remove retarget-io and printf in main function of cm33_ns

int main(void) { cy_rslt_t result = CY_RSLT_SUCCESS; /* Initialize the device and board peripherals. */ result = cybsp_init(); /* Board initialization failed. Stop program execution. */ if (CY_RSLT_SUCCESS != result) { __disable_irq(); /* Disable all interrupts. */ CY_ASSERT(0); while(true); /* Infinite loop */ } /* Enable global interrupts */ __enable_irq(); /**** retarget-io initialization and printf removed ****/ /* Enable CM55. */ /* CM55_APP_BOOT_ADDR must be updated if CM55 memory layout is changed.*/ Cy_SysEnableCM55(MXCM55, CM55_APP_BOOT_ADDR, CM55_BOOT_WAIT_TIME_USEC); for(;;) { Cy_GPIO_Inv(CYBSP_USER_LED1_PORT, CYBSP_USER_LED1_PIN); Cy_SysLib_Delay(BLINKY_LED_DELAY_MSEC); } }

- Now, initialise retarget-io in proj_cm55/main.c, and use printf to display a message

Code listing 4	Include retarget_io_init.h in cm55

/******************************************************************************* * Header File *******************************************************************************/ #include "cybsp.h" #include "retarget_io_init.h"

Code listing 5	Initialize retarget-io and use printf in main function of cm55

int main(void) { cy_rslt_t result; /* Initialize the device and board peripherals. */ result = cybsp_init(); /* Board init failed. Stop program execution. */ if (CY_RSLT_SUCCESS != result) { handle_app_error(); } /* Enable global interrupts. */ __enable_irq(); init_retarget_io(); /* \x1b[2J\x1b[;H - ANSI ESC sequence for clear screen */ printf("\x1b[2J\x1b[;H"); printf("********** PSOC Edge MCU: IPC Semaphore Example ********** \r\n"); printf("CM55 initialization \r\n\n"); /* Wait for UART traffic to stop */ while(!(Cy_SCB_UART_IsTxComplete(CYBSP_DEBUG_UART_HW))) {} /* Put the CPU to Deep Sleep. */ for (;;) { Cy_SysPm_CpuEnterDeepSleep(CY_SYSPM_WAIT_FOR_INTERRUPT); } }

- Build and program the application

### Output

After the program runs successfully, open the serial terminal and configure the baud rate to 115200-8-N-1. Then press the kit reset button.

Observe that the CM55 CPU prints the welcome message, while the CM33 CPU is still blinking the LED at approximately 1 Hz.

Figure 20	Output after using retarget-io on CM55

## Implement the GPIO button and print messages using both CPUs

In the previous section, we moved the retarget-io library to CM55; however, it is still possible for the CM33 to utilise the same UART. In this section, we will send UART messages from both CPUs when an EVK button is pressed.

- Open the Device Configurator from the Quick Panel area under the BSP Configurators section

- In the Device Configurator, go to the Pins tab

- In the PSOC™ Edge EVK user buttons are connected to P8.3 and P8.7, per the kit schematic in Figure 21

Figure 21	Connection of buttons in EVK schematics

- We will be utilizing USER BTN1 to print messages in our application, which is connected to P8.3.

- Rename Port8[3] as TRAINING_BUTTON

- Select File→Save or press Ctrl+S to save design.modus and regenerate source files.

Figure 22	Renaming P8[3] as TRAINING_BUTTON

- Open proj_cm33_ns/main.c and modify the code to use the new button alias to print a message to the terminal

Observe that we are using a UART PDL function since the CM33 project doesn’t include retarget-io. PDL functions for GPIO and UART, which are available in the mtb-dsl-pse8xxgp library.

Code listing 6	Update proj_cm33_ns/main.c to print UART message when button is pressed

for(;;) { if (Cy_GPIO_Read(TRAINING_BUTTON_PORT, TRAINING_BUTTON_PIN) == 0) { Cy_SCB_UART_PutString(CYBSP_DEBUG_UART_HW, "Message sent from CM33 \r\n"); } }

- Now, open proj_cm55/main.c and modify the code to implement a similar functionality

Observe that we are using printf for CM55 since retarget-io was included.

Code listing 7	Update proj_cm55 /main.c to print UART message when button is pressed

for (;;) { if (Cy_GPIO_Read(TRAINING_BUTTON_PORT, TRAINING_BUTTON_PIN) == 0) { printf("Message sent from CM55 \r\n"); } }

- Save the files, rebuild the project and program the device

### Output

After the program runs successfully, open the serial terminal and configure the baud rate to 115200-8-N-1. Then press the kit reset button.

Press USER_BTN1 on the EVK and observe that both devices will attempt to send messages to the terminal; however, the output is incorrect since no semaphores are implemented to synchronize access to UART.

Figure 23	Output sending messages from both CPUs without semaphores

## Use IPC semaphore to synchronise access to UART

We previously observed that both CPUs can access the UART concurrently; however, this can result in problems when both cores attempt to write at the same time without synchronisation.

In this section, we will use IPC semaphores to synchronise access.

- Create a shared folder to facilitate sharing parameters between projects.

- In Eclipse’s Project Explorer, right-click on the top level of the project and select New→Folder, then name the folder as shared

Figure 24	Create a shared folder

- Repeat the process to create an include folder inside shared folders should look as in Figure 25

Figure 25	Shared folders

- Right-click in the shares/include folder, and select New→File, then name the file as ipc_def.h

Figure 26	Create ipc_def.h

- Add the following code to the newly created ipc_def.h file

Code listing 8	ipc_def.h code

#ifndef IPC_DEF_H #define IPC_DEF_H /* IPC Channel to use */ #define IPC_CHANNEL_NUM 8U /* Semaphore number to be used in this example. Semaphores 0-15 are reserved for system use. */ #define MAX_SEMA_NUM 32U #define MY_SEMA_NUM 16U /* Delay to allow other core to acquire the semaphore */ #define SEMA_DELAY 10U #endif /* IPC_DEF_H */

- Add the shared/include path to proj_cm33_ns/Makefile and proj_cm55/Makefile

The Makefile for both projects needs to be updated since we will access ipc_def.h from both projects.

Code listing 9	Add shared include path to Makefile of proj_cm33_ns and proj_cm55

# Like SOURCES, but for include directories. Value should be paths to # directories (without a leading -I). INCLUDES+=../shared/include

- Open proj_cm33_ns/main.c and add code to initialize the semaphore

Observe that a region in shared memory is used to declare the semaphore variable since it needs to be accessed by both CPUs.

Code listing 10	Include shared file and declare sema_data in proj_cm33_ns/main.c

/******************************************************************************* * Header Files *******************************************************************************/ #include "cybsp.h" #include "ipc_def.h" /******************************************************************************* * Global variables *******************************************************************************/ CY_SECTION_SHAREDMEM uint32_t sema_data[CY_IPC_SEMA_COUNT / CY_IPC_SEMA_PER_WORD];

Code listing 11	Initialize semaphore in proj_cm33_ns/main.c

int main(void) { cy_rslt_t result = CY_RSLT_SUCCESS; cy_en_ipcsema_status_t ipc_status; /* Initialize the device and board peripherals. */ result = cybsp_init(); /* Board initialization failed. Stop program execution. */ if (CY_RSLT_SUCCESS != result) { __disable_irq(); /* Disable all interrupts. */ CY_ASSERT(0); while(true); /* Infinite loop */ } /* Enable global interrupts */ __enable_irq(); /**** retarget-io initialization and printf removed ****/ ipc_status = Cy_IPC_Sema_Init(IPC_CHANNEL_NUM, CY_IPC_SEMA_COUNT, sema_data); if (CY_IPC_SEMA_SUCCESS != ipc_status) { __disable_irq(); /* Disable all interrupts. */ CY_ASSERT(0); while(true); /* Infinite loop */ } /* Enable CM55. */ /* CM55_APP_BOOT_ADDR must be updated if CM55 memory layout is changed.*/ Cy_SysEnableCM55(MXCM55, CM55_APP_BOOT_ADDR, CM55_BOOT_WAIT_TIME_USEC); Cy_SysLib_Delay(SEMA_DELAY); for(;;)

- Then, add the code to synchronize access to the UART

Code listing 12	Use IPC semaphore in proj_cm33_ns/main.c

for(;;) { if (Cy_GPIO_Read(TRAINING_BUTTON_PORT, TRAINING_BUTTON_PIN) == 0) { /* Acquire semaphore */ while (Cy_IPC_Sema_Set(MY_SEMA_NUM, false) != CY_IPC_SEMA_SUCCESS) ; Cy_SCB_UART_PutString(CYBSP_DEBUG_UART_HW, "Message sent from CM33 \r\n"); while(!(Cy_SCB_UART_IsTxComplete(CYBSP_DEBUG_UART_HW))) {} /* Release semaphore */ while (Cy_IPC_Sema_Clear(MY_SEMA_NUM, false) != CY_IPC_SEMA_SUCCESS) ; } }

- Now, open proj_cm55/main.c and add code to initialize the semaphore

Observe that the sema_data variable does not need to be declared again.

Code listing 13	Include shared file in proj_cm55/main.c

/******************************************************************************* * Header File *******************************************************************************/ #include "cybsp.h" #include "retarget_io_init.h" #include "ipc_def.h"

Code listing 14	Initalize semaphore in proj_cm55/main.c

int main(void) { cy_rslt_t result; cy_en_ipcsema_status_t ipc_status; /* Initialize the device and board peripherals. */ result = cybsp_init(); /* Board init failed. Stop program execution. */ if (CY_RSLT_SUCCESS != result) { handle_app_error(); } /* Enable global interrupts. */ __enable_irq(); ipc_status = Cy_IPC_Sema_Init(IPC_CHANNEL_NUM, 0UL, NULL); if(ipc_status != CY_IPC_SEMA_SUCCESS) { handle_app_error(); } init_retarget_io();

- Lastly, use the semaphore to synchronize access to UART

Code listing 15	Use IPC semaphore in proj_cm55/main.c

for (;;) { if (Cy_GPIO_Read(TRAINING_BUTTON_PORT, TRAINING_BUTTON_PIN) == 0) { /* Acquire semaphore */ while (Cy_IPC_Sema_Set(MY_SEMA_NUM, false) != CY_IPC_SEMA_SUCCESS) ; printf("Message sent from CM55 \r\n"); while(!(Cy_SCB_UART_IsTxComplete(CYBSP_DEBUG_UART_HW))) {} /* Release semaphore */ while (Cy_IPC_Sema_Clear(MY_SEMA_NUM, false) != CY_IPC_SEMA_SUCCESS) ; } }

- Save the files, rebuild the project, and reprogram the device

### Output

After the program runs successfully, open the serial terminal and configure the baud rate to 115200-8-N-1. Then press the kit reset button.

Press USER_BTN1 on the EVK and observe that both devices will send messages to the terminal and now that IPC semaphores are used to synchronize access, both cores can print the complete message successfully.

Figure 27	Output sending messages from both CPUs using IPC semaphores

## Conclusion

We successfully learned how to use the Library Manager to add and remove libraries on both CPUs, implemented code to access a shared resource, and synchronized that access with an IPC semaphore.

This demonstrates IPC’s effectiveness for coordinating access between processors, and it can also be used to exchange messages between them.

# Appendix A: Creating a PSOC™ Edge application in ModusToolbox™

The following steps show how to create a new project for PSOC™ Edge in ModusToolbox™.

- Open the ModusToolbox™ Dashboard 3.6 and launch the Eclipse IDE for ModusToolbox™

- If you have not installed ModusToolbox™ or Eclipse IDE, see Required development tools section.

Figure 28	Open Eclipse IDE for ModusToolbox™ from Dashboard

- Choose a workspace directory for your project. A folder will be created if the workspace does not exist.

- After selecting a suitable directory, click the Launch button

It is recommended to keep the workspace path short. Windows has a 260-character path length limit, and a long workspace path may cause build issues when creating some applications.

Figure 29	Choose workspace directory

- Select New Application from the Quick Panel. Alternatively, go to File> New> ModusToolbox™ application.
This will launch the Project Creator Tool

Figure 30	Create a new application

- Once Projector Creator is launched, you will be prompted to select a board support package (BSP).

- Select the KIT_PSE84_EVAL_EPC2 BSP under the PSOC™ Edge BSPs and click Next

BSPs are aligned with our development/evaluation kits; they provide files for basic device functionality. A BSP typically has a design.modus file that configures clocks and other board-specific capabilities. That file is used by the ModusToolbox™ configurators. A BSP also includes the required device support code for the device on the board. You can modify the configuration to suit your application. 
Observe that ModusToolbox™ also includes BSPs for PSOC™ Edge EPC4 devices (KIT_PSE84_EVAL_EPC4), and for the PSOC™ Edge AI Kit (KIT_PSE84_AI); however, this training is intended to be used with the KIT_PSE84_EVAL, which includes EPC2 devices by default.

Figure 31	Selecting BSP in Project Creator

- A new window opens to select an application. The Project Creator includes categories and a search field to make it easier to find and select different examples. Click the checkbox near the application, optionally rename the project, and then click Create. Figure 32 shows the creation of the PSOC™ Edge Hello World application under the Getting Started section

Windows has a 260-character path length limit. A long workspace path and/or a long project name may cause build issues when creating some applications.

Figure 32	Selecting application

- A new project is being created, as shown in Figure 33, all the files in the Project Explorer window are displayed

Figure 33	New application created

# Appendix B: KIT_PSE84_EVAL details

28 Linear potentiometer (R34)

29 Analog microphones (IM73A135V01XTSA1, U36 and U37)**

30 User LEDs (D3, D4, D5)

31 Thermistor (TH1)

32 CAPSENSE™ buttons and slider (CSB1, CSB2, CSS1)

33 BOOT configuration switch (SW6)

34 Proximity sense connector (J19)

35 I/O headers compatible with Arduino UNO R3 (J2, J3, J4)

36 Alternative serial interface I/O header (J14)*

37 Power header compatible with Arduino UNO R3 (J1)

38 PSOC™ Edge E84 MCU expansion I/O headers (J6, J7, J40)*

39 MicroSD card holder (J35)**

40 Infineon Shield2Go interface headers (J10, J12)*

41 PDM microphones (IM72D128V01XTMA1, U7 and U8)**

42 mikroBUS compatible headers by Mikroelektronika (J9, J17)*

43 Extended I2S header (J11)*

44 6-axis accelerometer and gyroscope IMU (U5)

45 M.2 (E-key) radio interface connector (J13)

46 PSOC™ Edge E84 MCU power selection/monitoring headers (J18, J22, J23, J24, J25, J26)

47 Headphone connector (J34)*

48 Speaker (ACC6)

49 RJ45 ethernet magjack connector (J5)*

50 KitProg3 programming mode selection button (SW3)

1 Baseboard power LED (D1)

2 KitProg3 program/debug USB Type-C connector (J8)

3 PSOC™ 5LP-based KitProg3 programmer and debugger (CY8C5868LTI-LP039, U2)

4 Reset button (SW1)

5 KitProg3 status LED (D2)

6 PSOC™ Edge E84 MCU ETM/JTAG debug and trace header (J15)

7 PSOC™ Edge E84 MCU 10-pin SWD/JTAG program and debug header (J16)

8 Alternative serial interface configuration headers (J20, J21)

9 PSOC™ Edge E84 MCU USB host Type-A connector (J27)

10 USB Type-C Power Delivery (PD) fault LED (D6)

11 Custom display capacitive touch panel connector (J37)**

12 PSOC™ Edge E84 MCU USB device Type-C connector (J30)

13 External power supply VIN connector (J31)

14 PSOC™ Edge E84 MCU user buttons (SW2, SW4)

15 M.2 (B-key) memory interface connector (J29)

16 128 Mb Octal-SPI HYPERRAM™ (S70KS1283GABHI020, U12)***

17 Processor system-on-module (SoM) 260-pin SODIMM connector (J28)

18 AIROC™ CYW55513 tri-band (Wi-Fi & Bluetooth®) combo radio (U3) section

19 Processor system-on-module (SoM) power LED (D3)

20 1 Gb Octal-SPI NOR flash (S28HS01GTGZBH1030, U10)***

21 128 Mb Quad-SPI NOR flash (S25FS128SAGMFB100, U11)

22 MIPI-DSI custom display connector (J38)**

23 PSOC™ Edge E84 MCU (PSE846GPS2DBZC4A, U1)

24 PSOC™ 4000T CAPSENSE™ Co-processor (U9)***

25 Raspberry Pi-compatible MIPI-DSI display connector (J39)**

26 3-axis magnetometer (U4)

27 Raspberry Pi compatible display capacitive touch connector (J41)**

Figure 34	PSOC™ Edge evaluation kit (EVK) details

*    Footprint only, not populated on the board

**   Component at the bottom side of the Baseboard

***  Component on the bottom side of the SoM

Revision history

| Document revision | Date | Description of changes |
| --- | --- | --- |
| ** | 2026-03-15 | Initial release. |

Disclaimer