Training Manual for Introduction to PSOC™ Edge E84
Getting Started with the Development Ecosystem

About this document

Scope and purpose

This document is a training manual for the PSOC™ Edge E84 MCU. This Manual helps in getting started with PSOC™ Edge E84 MCU applications using Modus Toolbox™ software

Table of contents

Contents

About this document	1

Table of contents	1

1	Introduction	2

2	Required development tools	2

3	Prerequisites	2

4	Code Example 1: Hello world + Blinky	3

4.1	Objective	3

4.2	Description	3

4.3	Flowchart	3

4.4	Hardware Block Diagram and Setup	5

4.5	Project Creation	6

4.6	Conclusion	32

5	Code Example 2: IPC Semaphore using PDL	33

5.1	Objective	33

5.2	Description	33

5.3	IPC Semaphore Code Example Flowchart	33

5.4	Hardware Setup	35

5.5	Project Creation	35

5.6	Conclusion	56

# Introduction

- This manual provides specific instructions to create, configure , build and run several application code examples on the PSOC™ Edge E84.

# Required development tools

- ModusToolbox™ software v3.6 or later (tested with v3.6)

  - ModusToolbox™ Programming Tools v1.6.0

- Board support package (BSP) minimum required version for:

  - KIT_PSE84_EVAL_EPC2: v1.2.0

- PSOC™ Edge E84 Evaluation Kit (KIT_PSE84_EVAL Rev G)

  - Ensure BOOT SW (SW6) should be in 'High'/ON position.

    - Note: For KIT_PSE84_EVAL rev. G, BOOT SW pin is BOOT.1 (P17.6)

  - Ensure J20 and J21 should be in Tristate/Not-Connected (NC) position.

- Serial Terminal such as Tera Term, PuTTY, or the Serial Terminal in the Eclipse IDE

# Prerequisites

- Introduction to PSOC™ Edge E84 (E1) video training session. Note you must be logged into Infineon.com for this link to the video traing session to work.

- Make sure the required development tools are installed.

# Code Example 1: Hello world + Blinky

## Objective

This code example will teach you to create a simple UART communication by printing the “Hello World” message on a terminal and blinking an LED using a timer resource in PSOC™ Edge E84.  The code example uses the PDL (Peripheral Device Layer) API reference library.

## Description

This code example demonstrates the following algorithm flow:

Figure 1	Hello World + Blinky Algorithm Flow

## Flowchart

Below flow chart explains the flow of CM33 non-secure project

Figure 2	CM33_ns Flow Diagram

The below flow chart shows the flow of CM55 project, which is just to initialize the device and then enter a deepsleep.

Figure 3	CM55 Flow Diagram

## Hardware Block Diagram and Setup

Below is the Hardware block diagram for Hello world + Blinky example as per the schematic

UART pins:

- P6_5 (UART_RX)

- P6_7 (UART_TX)

GPIO LED Pin

- P16_7 (USER_LED)

Figure 4	Hardware Diagram for UART and LED Routing on PSOC™ Edge E84 EVK

  - Project Creation

- Before creating the first project we need to create our workspace in Modus™ Toolbox. Open Modus™ Toolbox from the Windows Start menu.

Note: if you have not installed Modus tool box, Refer Required development tools section

Figure 4	Launch Eclipse for ModusToolbox™

- Next, you must choose a workspace directory for your project. After selecting a suitable directory, click the Launch button.

Figure 5	Select workspace directory

- Hitting Launch will open a new ModusToolbox™ workspace as shown below.

Figure 6	Initial View of Eclipse workspace

- Now select New Application from the Quick Panel. Alternatively, go to File> New> Modus Toolbox™ Application

Figure 7	Create a New Application from the Quick Panel Start section

- Clicking New Application will open the Project Creator Tool and prompt you to choose a BSP (Board Support Package).

BSPs are aligned with our development/evaluation kits; they provide files for basic device functionality. A BSP typically has a design.modus file that configures clocks and other board-specific capabilities. That file is used by the ModusToolbox™ configurators. A BSP also includes the required device support code for the device on the board. You can modify the configuration to suit your application.

- Once Projector creator launches, Select the KIT_PSE84_EVAL_EPC2 BSP under the PSOC™ Edge BSPs (or Detected Devices if your EVK is already connected). Then click Next.

Figure 8	Choose BSP

- Clicking Next will take you to the next menu of the Project Creator which will ask you to select an Application. Select the PSOC Edge Empty Application under the Getting Started section. You can do this by selecting the checkbox near the application. Change the name to “PSOC_Edge_Hello_World” as we will be developing a hello world application and click on Create

Figure 9	Select Template Application – Getting Started – PSOC Edge Empty Application

- When you click Create, a new project is created, and you will be able to see all the files in the Project Explorer window.

Figure 10	 Project Explorer View

- Expaning the main project folder, you can see that there are multiple sub-folders within it.

Figure 11	Project folders of a PSOC Edge Applications with included FreeRTOS libraries

Exploring the folders, you will see the FreeRTOS libraries have been imported into the project. But, since we are making a basic hello world blinky project, the FreeRTOS library is not needed. We will remove it using the Library Manager tool. 

The Library Manager tool can be found in the Quick Panel tab directly below the Project Explorer in the Tools section. In addition, retargetIO will be needed for our project for using the UART functionalities. This will be added using the Library Manager as well.

Figure 12	Library Manager

- Select the PSOC_Edge_Hello_World project in the mtb project explorer and Open the Library Manager from Quick Panel. Find the freertos libraries under the proj_cm33_ns and proj_cm55 project libraries. Click the ‘X’ mark to remove the freertos library in each project’s libraries.

Figure 13	Removing freertos from cm33_ns and cm55

Now, click the Add Library option. A menu will open. Search for retarget-io. It can be found under the Peripheral section. Select the retarget-io option for proj_cm33_ns and click OK.

Figure 14	Adding retarget_io middleware

Now, click Update on the Library Manager.

Figure 15	Library Manager – Removing freertos and adding retarget-io middleware

Library Manager will update all the changes made to the hello world project. Close the Library Manager after the udpates have completed.

- Now let’s look how to configure or rename the pins using the Device Configurator.

Open the Device Configurator tool that can be found in the quick panel area under the BSP Configurators section.

The Device Configurator can be used to set up the system functions such as pins, interrupts, clocks, and DMA, as well as the basic peripherals, including UART, Timer, etc.

Figure 16	Selecting Device Configurator from Quick Panel – BSP Configurators

- After opening the Device Configurator go to the Pins section.

Figure 17	 Device Configurator - Pins

- In this particular Development Kit, the three User LEDs are connected to Port 16 Pins 5, 6 and 7 as per schematic

Figure 18	 Schematic View of PSE84 USER_LEDx Port connections

Each pin can be uniquely identified by giving them a name of your choice. These names are known as aliases and can be used in the code to refer to the pins instead of using the explicit Pin numbers.

Figure 19	Device Configurator – Pins – Port16 Resource – CYBSP_USER_LEDx Aliases

- For example, in Figure 20. Device Configurator - Pins – Creating Alias for USER_LED1 we have renamed Port 16 Pin 7 as TRAINING_LED. We will be using this LED in our application. Go to File > Save or alternatively hit Ctrl+S to save the change.

Figure 20. Device Configurator - Pins – Creating Alias for USER_LED1

Similarly let us look at the UART pins required for our PSOC_Edge_Hello_World code example. As per schematic Port 6 Pin 5 is used for UART_RX and Port 6 Pin 7 is used as UART_TX.

Figure 21	Pin assignments of UART_RX and UART_TX

Figure 22	Device Configurator > Pins > BSP UART Pin assignments

Remember these names “CYBSP_DEBUG_UART_TX” and “CYBSP_DEBUG_UART_RX”, as we will be using these during application development.

- Select File > Save to save our changes, and then Close the window to quit the configurator.

- The changes made in the Device Configurator can be see if you open the cycfg_pins.h file. The cycfg_pins.h file, along with all source files that get generated by the Device Configurator can be found GenerateSouce folder under with the application’s bsps folder tree. Figure shows the update. can see the pin definitions and aliases. And the change we have made has been reflected in the file as shown below.

Figure 23	Device Configurator - GeneratedSource folder location with cycfg_pins.h

Figure 24	Training_LED Alias in cycfg_pins.h

- In the project explorer you can see two folders named PSOC_Edge_Hello_World.proj_cm33_ns and PSOC_Edge_Hello_World.proj_cm55. Each of these folders will contain a main.c file which contains the application code. Replace these main.c files with main.c template files we have provided. (The template main.c files have required macros, definitions and basic initialization.

- Copy the contents from Lab_Source\templates\ Hello_World+Blinky \main_cm33_ns.c and paste into PSOC_Edge_Hello_World.proj_cm33_ns main.c

- Copy the contents from Lab_Source\templates\ Hello_World+Blinky \ main_cm55.c and paste on to PSOC_Edge_Hello_World.proj_cm55 main.c

Figure 25	cm33_ns main.c file location

- Remove “FREERTOS RTOS_AWARE” in the COMPONENTS from the proj_cm33_ns and proj_cm55 Makefile

Figure 26	Makefile for cm33_ns and cm55 – Remove component FREERTOS AWARE

- Build the projects and make sure nothing as broken at this point.

- Now let’s add the application code.

- Open the main.c file under PSOC_Edge_Hello_World.proj_cm33_ns. We will be adding the necessary code to this file for CM33 Core

Let us initialize the GPIO for Training LED. The Below code initializes the user LED pin. Copy and paste the below given code inside <code snippet> to the main.c file after __enable_irq();

Code listing 1	Initialize the GPIO using the PDL API to drive the TRAINING_LED

/* Set GPIO for use with LED */ Cy_GPIO_SetDrivemode(TRAINING_LED_PORT, TRAINING_LED_PIN, CY_GPIO_DM_STRONG);

The reference for PDL APIs will be available in PDL documentation can be found here, where you can search for particular function and get an overview of it. Here is an example for the Cy_GPIO_SetDrivemode we are using.

Figure 27	PDL API Online Reference - Cy_GPIO_SetDrivemode()

- Now let’s initialize the UART. First, paste in the global variables for the UART.

Code listing 2	Setting up Global Variables for the UART

static mtb_hal_uart_t DEBUG_UART_hal_obj; - static cy_stc_scb_uart_context_t DEBUG_UART_context;

Second, place the UART <code snippet>  below for use with retarget-io in the appropriate place i the main function.

Code listing 3	Initializing the UART for use with retarget_io middleware

/* Initialize UART for use with retarget_io*/ Cy_SCB_UART_Init(CYBSP_DEBUG_UART_HW, &CYBSP_DEBUG_UART_config, &DEBUG_UART_context); Cy_SCB_UART_Enable(CYBSP_DEBUG_UART_HW); result = mtb_hal_uart_setup(&DEBUG_UART_hal_obj, &CYBSP_DEBUG_UART_hal_config, &DEBUG_UART_context, NULL); /* UART setup failed. Stop program execution. */ if (CY_RSLT_SUCCESS != result) { handle_app_error(); } /*init retarget_io with uart*/ - cy_retarget_io_init(&DEBUG_UART_hal_obj);

- Now to allow for writing to the UART, we need to make sure global interrups are enabled.

Code listing 4	Enable global irqs to use UART

- __enable_irq();

- Since we have added code for the UART initialization, let’s add some print/logs to get printed to the terminal. Copy and paste the below given code inside <code snippet> to the main.c file

Code listing 5	Write Hello World header to UART

/* Print hello world */ printf("\x1b[2J\x1b[;H"); printf("****************** " "PSOC Edge MCU: Hello world using retarget-IO" "****************** \r\n\n"); printf("Hello World!!!\r\n\n"); printf("For more projects, " "visit our code examples repositories:\r\n\n"); printf("https://github.com/Infineon/" - "Code-Examples-for-ModusToolbox™™-Software\r\n\n");

- We have added several code snippets, so take a moment to build the cm33_ns project to make sure there are no build errors. There will be a few warnings for some variables we haven’t used yet, but there should be no build errors.

- Next, let’s focus on setting up to toggle the LED every 1 second. We need to setup a timer resource, so select the PSOC_Edge_Hello_World application in the project explorer and then select the Device Configurator in the Quick Panel > BSP Configurators section. Select the Peripherals tab, and open the Digital > Timer, Counter and PWM Group and select and used TWPCM 32-bit Counter

Figure 28	Device Configurator – Peripherals – Digital – Timer, Counter and PWM Group – Timer Selection

- Chosose the Timer – Counter 1.0 personality and click OK

Figure 29	Timer – Counter 1.0 Personality

- Rename the timer to TIMER

Figure 30	Naming the Timer

- Set up the Parameters for the TIMER

    - Run Mode to Continous

    - Period to 32000

    - Compare or Capture to Compare

    - Count Direction Up

    - Compare value to 32000

    - Check the box for Interrupt Source > Compare 0 & Capture 0

Figure 31	Timer Parameters

- Select the Input > Clock Signal. We can use the use the CYBSP_GENERAL_PURPOSE_TIMER_CLK_DIV.

Figure 32	Timer Input Clock Signal selection

- With the TIMER configured, choose File > Save or Ctrl+S and close the Device Configurator.

- Let’s place a call to timer_init() function which we will build later.

Code listing 6	Call to initialize the timer

/* Initialize timer to toggle the LED */ - timer_init();

- We will enable the second core as well using below code snippet. Copy and paste the below given code inside <code snippet> to the main.c file.

Code listing 7	Enable the CM55 CPU

/* Enable CM55 */ /* CM55_APP_BOOT_ADDR must be updated if CM55 memory layout is changed.*/ - Cy_SysEnableCM55(MXCM55, CM55_APP_BOOT_ADDR, CM55_BOOT_WAIT_TIME_USEC);

- Now Let’s add the Application code logic

  - First, we will be infinite for loop Polling for the key press entry in the terminal

  - Once User Presses the key, we will verify if the key is “carriage return(\r)”.

  - If It is carriage return, will check for “led_blink_active_flag”, if it set we will stop the timer and pause the LED Toggling

  - Else the LED Toggling Resumes

  - If the received key is not carriage return, we will check for “timer_interrupt_flag”, if it set we will Blink(toggle) the LED.

  - these a to f steps are executed in an infinite loop

As explained above, the code blinks the LED using the interrupts received from the Timer. It also checks whether the Enter key is pressed by the user via the UART and pauses or resumes LED blinking based on it.

Code listing 8	Main forever for loop for the cm33

/* main forever loop*/ for(;;) { /* Check to see if we have received anything on the uart*/ if (Cy_SCB_UART_GetNumInRxFifo(CYBSP_DEBUG_UART_HW) > 0) { scanf("%c", &uart_read_value); if (uart_read_value == '\r') { led_blink_active_flag = !led_blink_active_flag; if (led_blink_active_flag) { Cy_TCPWM_TriggerStart_Single(TIMER_HW, TIMER_NUM); printf("Enter key detected: Blinking Start\r\n"); } else { Cy_TCPWM_TriggerStopOrKill_Single(TIMER_HW, TIMER_NUM); printf("Enter key detected: Blinking Stops\r\n"); } } } /* Check if timer elapsed (interrupt fired) and toggle the LED */ if (timer_interrupt_flag) { /* Clear the timer irq flag */ timer_interrupt_flag = false; /* Toggle LED state */ Cy_GPIO_Inv(TRAINING_LED_PORT, TRAINING_LED_PIN); } - }

- The timer_init() function prototype was added to main.c, but we haven’t added the function yet. First, let’s set up the needed global variable.

Code listing 9	Global Variables for the timer irq configuration

cy_stc_sysint_t timer_irq_cfg = { .intrSrc = TIMER_IRQ, .intrPriority = TIMER_INT_PRIORITY - };

Second add the timer_init() function code below the timer_init summary.

Code listing 10	Function definition for the timer_init()

/******************************************************************************* * Function Name: timer_init ******************************************************************************** * Summary: * This function creates and configures a Timer object. The timer ticks * continuously and produces a periodic interrupt on every terminal count * event. The period is defined by the 'period' and 'compare_value' of the * timer configuration structure 'led_blink_timer_cfg'. Without any changes, * this application is designed to produce an interrupt every 1 second. * * Parameters: * none * * Return : * void * *******************************************************************************/ void timer_init(void) { /* Initialze TCPWM block with required timer configuration */ if (CY_TCPWM_SUCCESS != Cy_TCPWM_Counter_Init(TIMER_HW, TIMER_NUM, &TIMER_config)) { CY_ASSERT(0); } /* Set the interrupt line for COUNTER_HW */ if (CY_SYSINT_SUCCESS != Cy_SysInt_Init(&timer_irq_cfg, &timer_isr)) { CY_ASSERT(0); } /* setup the timer interrupt */ NVIC_EnableIRQ(TIMER_IRQ); /* Enable the initialized counter */ Cy_TCPWM_Counter_Enable(TIMER_HW, TIMER_NUM); /* Start the counter */ Cy_TCPWM_TriggerStart_Single(TIMER_HW, TIMER_NUM); }

- Now add the interrupt handler function for the timer. Copy and paste the below given code inside <code snippet> to the main.c file below timer_init summary.

Code listing 11	Function definition for the timer irq handler

/******************************************************************************* * Function Name: timer_isr ******************************************************************************** * Summary: * This is the interrupt handler function for the timer interrupt. * * Parameters: * callback_arg Arguments passed to the interrupt callback * event Timer/counter interrupt triggers * * Return: * void *******************************************************************************/ static void timer_isr(void) { uint32_t interrupts = Cy_TCPWM_GetInterruptStatusMasked(TIMER_HW, TIMER_NUM); Cy_TCPWM_ClearInterrupt(TIMER_HW, TIMER_NUM, interrupts); /* Check for Timer Count event */ if (BIT_MASK_CHECK != (CY_TCPWM_INT_ON_CC0 & interrupts)) { timer_interrupt_flag = true; } - }

- Save the file. We are done with adding the application code for PSOC_Edge_Hello_World.proj_cm33_ns

- Let’s add the application code for PSOC_Edge_Hello_World.proj_cm55. Open main.c under PSOC_Edge_Hello_World.proj_cm55 and put the CM55 core to system deep sleep, as we are not performing any task in this core. Copy and paste the below given code inside <code snippet> to the PSOC_Edge_Hello_World.proj_cm55 main.c after __enable_irq();

Code listing 12	Main forever for loop for the cm55

/* Put the CPU to deepsleep */ for (;;) { Cy_SysPm_CpuEnterDeepSleep(CY_SYSPM_WAIT_FOR_INTERRUPT); }

- Save the file. We are done with adding the application code to both cores

- Let’s build the Application by making sure we select the PSOC_Edge_Hello_World folder in the project explorer and then by clicking on the Build Application option in the Quick Panel.

Figure 33	PSOC_Edge_Hello_World Build Application

- After the Build process is successfully completed. We can program the board by clicking the Launch Program Application tool in the Quick Panel > Launches > PSOC_Hello_World.

Figure 34	PSOC_Edge_Hello_World Launch Program Application

- After programing is successful, Open a Serial Terminal inside the Eclipse IDE and configure with a baud rate 115200 and 8-N-1 then restart the EVK. You will see the message as shown in below image and the kit RED LED blinks at approximately 1 Hz. Press Enter Key to pause and resume the LED Blinking respectively.

Figure 35	Eclipse IDE Terminal Selection

Figure 36	Eclipse IDE Terminal Serial Configuration

Figure 37	Eclipse IDE Terminal Serial Communication from the EVK

  - Conclusion

We successfully created a PSOC™ Edge E84 application using ModusToolbox™, utilizing the Device Configurator for GPIO, UART and Timer, the Library Manager for adding the retarget-io middleware, and application code for print logs, key capture on a terminal, and a timer interrupt to blink the LED every 1 second.

# Code Example 2: IPC Semaphore using PDL

## Objective

This lab exercise demonstrates how to use the inter-processor communication (IPC) PDL driver to implement a simple semaphore scheme with PSOC™ Edge. A semaphore is used to synchronize start up and manage access to a common resource, a UART, between the CPUs.

## Description

In this code example, at device reset, the secure boot process starts from the ROM boot with the secure enclave (SE) as the root of trust (RoT). From the secured enclave, the boot flow is passed on to the system CPU subsystem where the secure CM33 application starts. After all necessary secure configurations, the flow is passed on to the non-secure CM33 application. Resource initialization for this example is performed by this CM33 non-secure project. It configures the system clocks, pins, clock to peripheral connections, and other platform resources. It then enables the CM55 core using the Cy_SysEnableCM55() function. After this, the cores execute their respective firmware.

The same semaphore is also used as a method to synchronize the access to a shared resource by the two CPUs. In this case, a UART is the resource. Both CPUs print a message to the computer terminal through UART every time the user button is pressed. Before printing a message, both CPUs attempt to lock the semaphore. If it succeeds, it prints the message and then unlocks the semaphore afterwards. If the semaphore is already locked, it keeps trying to lock it until it succeeds.

The firmware has an option to disable the semaphore in the code. The semaphore is no longer locked by the CPUs by simply changing #define ENABLE_SEMA to 0U in the ipc_def.h file; additionally, the kit LED turns ON. Without the semaphore to synchronize access to the UART, messages from both CPUs might overlap. See the flowchart to see visually how the code will operate.

## IPC Semaphore Code Example Flowchart

Figure 38	IPC Semaphore Code Example Flow

## Hardware Setup

Figure 39	Hardware setup with USB cable for programming and serial UART

## Project Creation

- Since we are already familiar with the project creation process, we will start by creating a New Application.

- We will start with a PSOC™ Edge Empty Application and turn it into an PSOC™ Edge IPC Semaphore using PDL application.

- Click New Application from the Quick Panel.

Figure 40	Start a New Application

- Clicking New Application will open the Project Creator Tool and prompt you to choose a BSP (Board Support Package).

- Once Projector creator launches, Select the KIT_PSOCE84_EVK_EPC2 BSP under the PSOC™ Edge or Detected Devices BSPs. Then click Next.

Figure 41	Select BSP

- Clicking Next will take you to the next menu of the Project Creator which will ask you to select an application. Select the PSOC Edge Hello World under the Getting Started section. You can do this by selecting the checkbox near the application. Change the name to “PSOC_Edge_IPC_Semaphore” as we will be developing the Hello World application into a dual-core IPC application.

- Click on Create.

Figure 42	Select Template Application – Rename Application - Create

- If you expand the main project folder, you can see that there are multiple sub-folders within it.

Figure 43	Project Explorer View

The Hello_World project uses the retargetIO in proj_cm33_ns. We will remove it the cm33_ns project using the Library Manager tool. The Library Manager can be found in the Quick Panel > Tools menu. Also, retarget-io is needed for our project for using the UART functionalities in proj_cm55. This too can be added using the Library Manager.

Figure 44	Opening the Library Manager from the Quick Panel

- Select the PSOC_Edge_IPC_Semaphore Project in the mtb project explorer and open the Library Manager from quick panel. You will be able to see the retarget-io within the proj_cm33_ns libraries group. Click the ‘X’ icon to remove the library.

Figure 45	Library Manager – Remove retarget-io from cm33_ns project

Now, click the Add Library option. A menu will open. Change the Target Project to proj_cm55 and Search for retarget-io. It will be found under the Peripheral section. Select the retarget-io option and click OK.

Figure 46	Library Manager – Adding retarget-io middleware to the cm55 project

Now, click Update on the Library Manager.

Figure 47	Library Manager – Update changes to retarget-io

- Library Manger will update all the changes made to the PSOC_Edge_IPC_Semaphore application.

- Since we removed the retarget_io middleware from the cm33_ns project and add it to the cm55 project, we also need to move the files retarget_io_init.c and retarget_io_init.h from the PSOC_Edge_IPC_Semaphore.proj_cm33_ns folder to the PSOC_Edge_IPC_Semaphore.proj_cm55 folder.

Back in the Project Explorer of the Eclipse IDE, open the PSOC_Edge_IPC_Semaphore.proj_cm33_ns folder and right click on the retarget_io_init.c file and select Move…

Figure 48	Moving retarget_io_init.c and .h from cm33_ns project o cm55 project

Then select the cm455 project folder as the desitination folder

Figure 49	Select the PSOC_Edge_IPC_Semaphore.proj_cm55 folder as the destination for retarget_io_init.c

Do the same procedure for the retarget_io_init.h file, moving it to the PSOC_Edge_IPC_Semaphore.proj_cm55 folder. In the Project Explorer, verify that both files are now located in the PSOC_Edge_IPC_Semaphore.proj_cm55 folder as shown in the figure below

Figure 50	Verfication of moving retarget_io_init.c and .h to the cm55 project folder

- Now we need to replace the starter code for the main.c files in folders named PSOC_Edge_IPC_Semaphore.proj_cm33_ns and PSOC_Edge_IPC_Semaphore.proj_cm55. Each of these folders contain a main.c file which contains the application starter code. Replace the code in these main.c files with the code from the main.c template files we have provided. (The template main.c files have required Macros, definitions and basic initialization.)

- Copy the contents from Lab_Source\templates\ PSOC_Edge_IPC_Semaphore \main_cm33_ns.c and paste into PSOC_Edge_IPC_Semaphore.proj_cm33_ns main.c replacing the default code.

- Copy the contents from Lab_Source\templates\ PSOC_Edge_IPC_Semaphore \main_cm55.c and paste into PSOC_Edge_IPC_Semaphore.proj_cm55. main.c replacing the code.

- After replacing the code in both main.c file, make sure to Save them both, and the Build the Application and make sure nothing as broken at this point. Remember to select the PSOC_Edge_ICP_Semaphore application folder before using the Quick Panel > PSOC_Edge_IPC_Semaphore (APP_KIT_PSE84_EVAL_EPC2) > Build Application command.

- Now let’s add the application code.

- In Windows Explorer (or the equivalent folder explorer on your OS), copy the shared folder from Lab_Source\templates\PSOC_Edge_IPC_Semaphore\ , navigate to the workspace folder on your local drive, locate the application folder for PSOC_Edge_IPC_Semaphore and paste the shared folder there.

This shared folder contains the “ipc_def.h” file, which contains definitions of constants and structures for setting up user semaphores and function prototypes for configuring the IPC subsystem.

Figure 51	Placing a copy of the shared folder into the PSOC_Edge_IPC_Semaphore application folder

- In order for the build system to be able to utilize the shared folder we need to modify the Makefile to include the “ipc_def.h” file folder location. In the Project Explorer, open the Makefile for project PSOC_Edge_IPC_Semaphore.proj_cm33_ns

Figure 52	Opening the cm33_ns project Makefile

- Inside the Makefile, navigate to the Advance Configuration section and change the INCLUDES= directive to to INCLUDES=../shared/include and Save and Close the file

Figure 53	Modifying the INCLUDES directive of the cm33_ns project’s Makefile

**Make sure to make the same change to the INCLUDES directive in the Makefile for the cm55 project as both projects will be utilizing the ipc_def.h header file.**

- Open the main.c file under PSOC_Edge_IPC_Semaphore.proj_cm33_ns. We will be adding the necessary code to this file for CM33 Core.

Figure 54	Opening the cm33_ns project main.c file from the Project Explorer

- Add IPC communication header file to the main.c

- Copy and paste the code provided within the Code listing below to the main.c file after other #includes near the beginning of the file.

Code listing 13	Include ipc_def.h in cm33_ns main.c

- #include "ipc_def.h"

- Add the ipc_status variable in the main function

Copy and paste the code provided within the Code listing below to the main function after the associated comment /* Initialize IPC subsytem */

Code listing 14	Local variable to main function for ipc_status

- cy_en_ipcsema_status_t ipc_status;

- Add the code to initialize the IPC semaphore subsystem

Copy and paste the code provided below within the Code listing within to the main function after the associated comment /* variable to hold ipc_status*/

Code listing 15	Code snippet for initializing IPC Semaphore Subsystem

- ipc_status = Cy_IPC_Sema_Init(IPC_CHANNEL_NUM, CY_IPC_SEMA_COUNT, sema_data); - if (CY_IPC_SEMA_SUCCESS != ipc_status) - { - /* Disable all interrupts. */ - __disable_irq(); - CY_ASSERT(0); - /* Infinite loop */ - while(true); - }

- Have the cm33_ns code lock the semaphore in preparation of starting up the cm55

Copy and paste the code provided below within the Code listing within to the main function after the associated comment /* lock IPC semaphore SEMA_NUM*/

Code listing 16	Code snippet for locking the  IPC Semaphore SEMA_NUM

- Cy_IPC_Sema_Set(SEMA_NUM, false);

- The cm33_ns application has enabled the cm55 CPU, so now we need to wait until the cm55 initializes and releases/unlocks the semaphore.

Copy and paste the code provided below within the Code listing within to the main function after the associated comment /* Wait for cm55 to finish startup and release the semaphore*/

Code listing 17	Code snippet for wait for cm55 to complete initialization

- while(Cy_IPC_Sema_Status(SEMA_NUM) == CY_IPC_SEMA_STATUS_LOCKED) {}

- With the cm33 setup and the cm55 having completed its initialization we enter the main forever for loop. In the loop we will

- Check if the user has pressed the USER_BTN1

- If the we detect a button press

  - Send the cm33 message to the UART

  - Set/lock the semaphore SEMA_NUM

  - Wait for the cm55 to clear/unlocked the semaphore SEMA_NUM

Copy and paste the code provided below within the Code listing within the infinite for loop at the end of the main function

Code listing 18	Code snippet for cm33_ns main forever for loop

- /* If there is a user button press */ - if (Cy_GPIO_Read(CYBSP_USER_BTN_PORT, CYBSP_USER_BTN_PIN) == 0) - { - /* Send cm33 message to the UART */ - Cy_SCB_UART_PutString(CYBSP_DEBUG_UART_HW, "Message sent from CM33 \r\n"); - #if ENABLE_SEMA - /* Lock the semaphore */ - Cy_IPC_Sema_Set(SEMA_NUM, false); - /* Wait for CM55 to release the semaphore */ - while(Cy_IPC_Sema_Status(SEMA_NUM) == CY_IPC_SEMA_STATUS_LOCKED) {} - #endif - }

- This is a good time to make sure the application still builds without errors. Remember to select the PSOC_Edge_ICP_Semaphore application folder before using the Quick Panel > PSOC_Edge_IPC_Semaphore (APP_KIT_PSE84_EVAL_EPC2) > Build Application command.

- Now that the application is building without errors, we can move on to the cm55 project. In the Project Explorer, open the PSOC_Edge_IPC_Semaphore.proj_cm55 folder and double click the main.c file in this folder to open main.c

Figure 55	Open main.c in the PSOC_Edge_IPC_Semaphore.proj_cm55 project folder

- Like we did for the cm33_ns project, add the IPC communication header file to the main.c

- Copy and paste the code provided within the Code listing below to the main.c file after other #includes near the beginning of the file.

Code listing 19	Include ipc_def.h in cm55 main.c

- #include "ipc_def.h"

- Now, let’s add the Macro for the LED_STATE.

Copy and paste the code provided within the Code listing below to the Macros section of the file after the associated comment /* Define the state of the USER_LED based on whether a semaphore is used*/

Code listing 20	Local variable to main function for ipc_status

- #if ENABLE_SEMA - #define LED_STATE CYBSP_LED_STATE_OFF - #else - #define LED_STATE CYBSP_LED_STATE_ON - #endif

- Add the ipc_status variable in the main function

Copy and paste the code provided within the Code listing below to the main function after the associated comment /* Initialize IPC subsytem */

Code listing 21	Local variable to main function for ipc_status

- cy_en_ipcsema_status_t ipc_status;

- Next, we need to add code for the cm55 to initialize the IPC subsystem. Note that the cm33 already setup the default semaphore count for the channel and a shared memory buffer for the IPC subsystem, so the cm55 will initialize with 0 count and a NULL pointer. Below is an excerpt from the IPC PDL Semaphore layer initialization documentation.

Figure 56	DSL > PDL > IPC > IPC semaphores layer > Functions > Cy_IPC_Sema_Init() Documentation

Copy and paste the code provided below within the Code listing within to the main function after the associated comment /* Initialize IPC subsytem */

Code listing 22	Code snippet for cm55 initialization of the IPC Semaphore Subsystem

- ipc_status = Cy_IPC_Sema_Init(IPC_CHANNEL_NUM, 0UL, NULL); - if(ipc_status != CY_IPC_SEMA_SUCCESS) - { - handle_app_error(); - }

- Let’s have the cm55 send out the IPC Semaphore Example header and then wait for the UART to complete transmitting the message.

Copy and paste the code provided within the Code listing below to the main function after the associated comment /* Send Header message out to UART*/

Code listing 23	Header message for the IPC Semaphore example to UART from cm55

- /* \x1b[2J\x1b[;H - ANSI ESC sequence for clear screen */ - printf("\x1b[2J\x1b[;H"); - printf("****************** PSOC Edge MCU: IPC Semaphore Example ****************** \r\n\n"); - printf("<Press the kit's user button to print messages>\r\n\n"); - /* Wait for UART traffic to stop */ - while(!(Cy_SCB_UART_IsTxComplete(CYBSP_DEBUG_UART_HW))) {}

- With the initial header message sent to UART, let’s set the USER_LED according to whether the build includes the code within the #if ENABLE_SEMA or not. Note that when the LED is illuminated the main loop is running without semaphore control.

Copy and paste the code provided within the Code listing below to the main function after the associated comment /* Turn on LED when Semaphores are not being used*/

Code listing 24	Set GPIO to LED_STATUS indicating the use of semaphones or not

- Cy_GPIO_Write(CYBSP_USER_LED1_PORT, CYBSP_USER_LED1_PIN, LED_STATE);

- Now that the cm55 setup is complete, let’s have the cm55 release/unlock the semaphore SEMA_NUM

Copy and paste the code provided within the Code listing below to the main function after the associated comment /* unlock IPC semaphore SEMA_NUM*/

Code listing 25	Release/unlock the semaphore SEMA_NUM

- Cy_IPC_Sema_Clear(SEMA_NUM, false);

- With the cm55 setup and having released/unlocked the semaphore letting  the cm33_ns code know that the cm55 has completed its start up tasks, we enter the main forever for loop. In the loop we will

- Check if the user has pressed the USER_BTN1

- If the we detect a button press

  - Wait for the cm33 to set/lock the semaphore SEMA_NUM

  - Send the cm55 message to the UART

  - Clear/unlock the semaphore SEMA_NUM

Copy and paste the code provided below within the Code listing within the infinite for loop at the end of the main function after the associated comment /* If there is a user button press */

Code listing 26	Code snippet for cm55 main forever for loop

- if (Cy_GPIO_Read(CYBSP_USER_BTN_PORT, CYBSP_USER_BTN_PIN) == 0) - { - #if ENABLE_SEMA - /* Wait for CM33 to lock the semaphore */ - while(Cy_IPC_Sema_Status(SEMA_NUM) == CY_IPC_SEMA_STATUS_UNLOCKED) {} - #endif - /* Send cm55 message to the UART */ - printf("Message sent from CM55 \r\n"); - #if ENABLE_SEMA - /* Release semaphore */ - Cy_IPC_Sema_Clear(SEMA_NUM, false); - #endif - }

- Save the file. We are done with adding the application code for both the cores.

- Let’s build by making sure the PSOC_Edge_IPC_Semaphore application folder it selected and then clicking on the Build Application option in the Quick Panel.

Figure 57	Building the PSOC_Edge_IPC_Semaphore application

- After the Build process is successfully completed. We can program the board by clicking the using the  Quick Panel > Launches > PSOC_Edge_IPC_Semaphore Program Application button.

Figure 58	Programming the PSOC_Edge_IPC_Semaphore application to the target EVK

- After programing is successful, Open a Serial Terminal inside the Eclipse IDE and configure with a baud rate 115200 and 8-N-1 then restart the EVK. You will see the message as shown in below.

Figure 59	Eclipse IDE Terminal Selection

Figure 60	Eclipse IDE Terminal Serial Configuration

Figure 61	Eclipse IDE Terminal Serial Communication from the EVK showing UART IPC Header

- Confirm the USER_LED is not illuminated, indicating the application was built utilizing a semaphore to control access to the UART. Press the USER_BTN1, and observe the messages in the serial terminal window. Since a semaphore is being used, no mixing of messages is obsevered and the messages alternate cleanly between the cm33_ns and cm55 code bases.

Figure 62	Clean UART messages from cm33 and cm55 when using a semaphore

- Now, let’s turn off the use of semaphores to manage access to the shared UART, build the application and the observe the results on the serial terminal.

Using the Project Explorer navigate to and open the ipc_def.h file

Figure 63	Open the shared > include > ipc_def.h header file

Scroll down and find the ENABLE_SEMA macro and change its value from 1U to 0U

Figure 64	Update ENABLE-SEMA macro value to 0

Save and Close the file. Select the PSOC_Edge_IPC_Semaphore application folder, then build and launch the application

- Confirm the USER_LED is now illuminated, indicating the application was built without utilizing a semaphore to control access to the UART. Press the USER_BTN1, and observe the messages in the serial terminal window. Since a semaphore is no longer being used, we now observe mixing of the messages as they get transmitted by both the cm33_ns and cm55 code bases.

Figure 65	Mixed UART messages from cm33 and cm55 when not using a semaphore

  - Conclusion

We successfully created a PSOC™ Edge E84 IPC PDL Semaphore application using ModusToolbox™, figured out how to use IPC PDL Libraries and access the same resource, in this example the debug UART, from two cores using the IPC PDL Semaphore layer.